SECTION 23 07 13

EcoVerbTM Recycled Cotton Acoustical Duct Liner

PART 1 GENERAL

- SECTION INCLUDES

  - EcoVerbTM recycled cotton acoustical duct liner roll and accessories for HVAC installation.

- REFERENCES

  - American Society for Testing and Materials (ASTM)

    - ASTM E84: Standard Test Method for Surface Burning Characteristics of Building Materials.

    - ASTM C423: Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method.

    - ASTM E795: Standard Practices for Mounting Test Specimens During Sound Absorption Tests

  - International Code Council (ICC)

    - ICC IBC: International Building Code

- SUBMITTALS

  - Product Data: Manufacturer’s data sheet and installation instructions.

  - Samples: Submit, at minimum, a 4” x 4” sample for each type of specified duct liner.

  - Test Reports: Upon request, submit certified test reports to verify specified product performance.

- QUALITY ASSURANCE

  - Qualifications:

    - Manufacturers: Provide acoustical duct liner from a single manufacturer.

    - Installers: Utilize an installer having demonstrated experience on projects of comparable size and complexity.

  - Performance Requirements:

    - Surface Burning Characteristics: Acoustical product to perform as specified when tested in accordance with ASTM E84. Acoustical products surface burning performance should comply with the International Building Code and other local building code requirements.

    - Acoustical Characteristics: Acoustical products to perform as specified when tested in accordance with ASTM C423.

- DELIVERY, STORAGE, AND HANDLING

  - Storage and Handling Requirements:

    - Handle duct liner carefully to avoid any damage.

    - Store duct liner indoors in a clean, cool, dry place, and out of direct sunlight.

    - Store duct liner in a space where the ambient temperature and humidity conditions are being maintained at the levels indicated for the project when occupied for its intended use.

- SITE CONDITIONS

  - Ambient Conditions:

    - Maintain ambient temperature and humidity conditions at levels indicated for the project when occupied for its intended use.

    - Do not install liner under environmental conditions outside manufacturer’s recommended limits.

  - Existing Conditions: Do not install duct liner until space is enclosed and weather proofed, and wet work is completely dry.

- WARRANTY

  - Provide manufacturer’s written product warranty per Section 01 77 00 – Closeout Procedures.

PART 2 PRODUCTS

(Specifier Note: Red colored text below requires you to select an option before this specification can be completed. Options in red text are bound by parenthesis. Additional EcoVerb Roll information can be found on the EcoVerb Roll Product Page of our website.)

- MANUFACTURERS

  - Second Skin Audio, 13825 W Business Center Drive STE B, Lake Forest, IL 60045.
Phone: 800-679-8511. Website: www.secondskinaudio.com

- DESCRIPTION

  - Product: EcoVerbTM recycled cotton acoustical duct liner roll by Second Skin Audio

  - Product Options:

    - Composition:		Recycled bonded acoustical cotton with black fire-resistant facing

    - Density:		(1.5lb pcf) / (3lb pcf – 1/2" thickness only)

    - Thickness:		(1/2”) / (1”) / (1-1/2”) / (2”)

    - Size:			1/2”: 		48” width x (up to 100’ length) roll
 			1”		48” width x (up to 50’ length) roll
 			1-1/2” and 2”:	48” width x (up to 25’ length) roll

    - Finish:			Marble blue with black facing

    - Mounting Method:	Adhesive

  - Product Performance:

    - Acoustical Performance

      - Noise Reduction Coefficient (NRC) per ASTM C423

| Thickness/ Density | 1/2" – 3lb pcf | 1” – 1.5lb pcf | 1-1/2” – 1.5lb pcf | 2” – 1.5lb pcf |
| --- | --- | --- | --- | --- |
| A Mount | 0.40 | 0.75 | 0.85 | 1.15 |

    - Surface Burning Performance

      - Fire Rating per ASTM E84: Class A.

- ACCESSORIES

- Attachment hardware for duct liner as specified by manufacturer for installation.

PART 3 EXECUTION

- EXAMINATION

  - Verification of Conditions:

    - All wet work in the installation area must be complete, cured, and dry prior to installation.

- INSTALLATION

- Comply with manufacturer’s instructions and recommendations for installation of duct installation and with industry standards.

- PROTECTION

- Protect installed work from damage due to subsequent construction activity, including temperature and humidity limitations and dust control, so that the work will be without damage and deterioration at the time of acceptance by the owner.

END OF SECTION